GNRX320

Ten-Tec RX-320 Control Program

Version 1.30

March 2002

Gerd Niephaus

Contents

1)	Introduction	3

2)	Installation	5

1)	New installation	5

2)	Replacement of an existing installation	6

3)	Reference	7

1)	Frequency group	7

2)	Mode group	7

3)	AGC group	7

4)	Filter group	7

5)	Passband shift group	7

6)	Signal strength group	8

7)	Station group	8

8)	Volume group	8

9)	Line out group	8

10)	Display of time and date	9

11)	Station file button	9

12)	PDB button	9

13)	Configure button	9

14)	Exit button	9

4)	Controling the program via keyboard	10

5)	Appendix	11

1)	Format of the station file	11

2)	Configuration of the lowest tunable frequency	12

# Introduction

The program GNRX320 is intended to operate the Ten-Tec RX-320 receiver. The program requires Microsoft Windows 95, 98, ME, NT, 2000 or XP; it will not work with Windows 3.11 or DOS.

GNRX320 uses only standard Windows user controls; it therefore does not require special OCX or dynamic link libraries to be installed. As a consequence the installation is extremly simple and (hopefully) robust. The distribution archive is very small.

The program makes all the hardware features of the Ten-Tec RX-320 receiver available to the user:

- 34 different filter bandwidths,

- 3 different AGC speeds,

- smallest tuning step size is 1 Hz,

- selectable passband shift in LSB and USB mode.

The features of the program may be summarized as:

- Tuning with Up and Down buttons or optional continuous tuning with 12 different tuning step sizes. Two different tuning speeds selectable. Direct frequency input via keyboard is possible.

- All the available operation modes and AGC modes can be chosen.

- Filter bandwidth accessible in 4 groups of 9 different filters each.

- Passband shift can be varied in a wide range; it can easily be switched off.

- Signal strength is displayed as actual value and average value. The display can be switched off.

- Squelch control is implemented; the squelch state is displayed.

- Audio output level and line output level can be controlled and switched off separately. Mute / don’t mute on exit can be chosen.

- Display of current date and UTC time.

- Important system parameters can easily be configured by the user.

- A simple memory facility is implemented: Frequency, station name, mode, AGC, filter and passband shift can be stored for an arbitrary number of stations. The stations can be organized into different station files according to the users preferences. There are possibilities to store the actual radio settings, edit, update and delete the station entries and tune the receiver to the selected station entry.

- Almost all functions of the control program can be invoked via keyboard input i.e without use of a mouse.

- A link to the ILG broadcast database program GNPDB has been incorporated. It is now possible to tune the receiver from within the database program and to look up the frequency the receiver is tuned to in the ILG database.

- According to suggestions by Bill Farmer, W3CSW, the tuning in CW mode has been enhanced.

- A link to the Klingenfuss ”Super Frequency List” (i.e. a broadcast and utility station database available on CD) has been incorporated. This link is established via the separate program GNKFDB.

- The actual window position and state (minimized/normal size) will now be stored when exiting the program. This position and  state will be restored when the program is started the next time.

- Frequency correction is now calculated via interpolation assuming linear frequency dependence.

- The mouse wheel may be used for the input of the tuning frequency.

# Installation

## New installation

The program GNRX320 will work with MS Windows 95, 98, ME, NT, 2000 and XP. No special hardware is required.

The version of the Windows dll COMCTL32.dll must be 4.70 or later. If this is not true, problems may be encountered with the station display.

Installation of the Ten-Tec RX-320 Control Program GNRX320 is quite easy. The software is distributed as a zip archive:

- gnrx320130.zip  (”130” is the actual version number)

Extract gnrx320130.zip in a directory where you want the software to reside. This will produce the following files in the directory:

- gnrx320.exe 	(the program)

- gnrx320.doc	(program description in MS Word format)  (this file)

- license.txt		(license and disclaimer)

- readme.txt		(conditions of use)

- changes.txt	(program history)

When the program (gnrx320.exe) is started the first time, it will ask for the number of the COM-port the Ten-Tec RX-320 receiver is connected to.

This is the complete installation! The software does not modify the registry nor does it install other files (except the ini-file; see below).

In order to simplify starting of the program, you can of course manually create a shortcut to the program on your desktop.

The system remembers all the settings of its last use. These settings are stored in a file

- gnrx320.ini

which resides in the Windows system directory.

If you want to remove the software from your system completely, delete the files just mentioned as well as all the station files you may have generated (see Reference part of this manual).

## Replacement of an existing installation

Delete the files

- gnrx320.exe

- gnrx320.doc

- license.txt

- readme.txt

- changes.txt

in your existing installation directory. Extract the download file (gnrx320130.zip) in this directory. This completes the replacement.

Version 1.30 is completely compatible with the versions 1.00, 1.10, 1.20 and version1.21. So you can use your existing station files as well as the file gnrx320.ini without conversion or modification.

# Reference

The user interface of the Ten-Tec RX-320 Control Program GNRX320 consists of several

control elements. These controls are grouped into logical groups in order to simplyfy the operation of the program.

## Frequency group

The leftmost display in this group shows the frequency the receiver is tuned to. This frequency may be shown in MHz or KHz depending on the user configuration (see ”Configure” button).

The ”Up” and ”Down” buttons will change the frequency by the step size which has been chosen through the radio buttons at the bottom of the frequency group. If you hold the ”Up” or ”Down” button down, the system will start to continuously alter the frequency by the step size. There are two stepping rates available which can be chosen with the ”Fast” button.

The ”ContUp” and ”ContDown” buttons are similar to the ”Up” and ”Down” buttons. They allow continuous tuning with the chosen stepping rate without holding the buttons pressed down. Clicking the respective button again will stop the stepping.

The edit box to the right of the frequency group allows for direct input of the desired frequency using the keyboard. Clicking the ”Enter” button will tune the receiver to the frequency visible in this edit box. Input may be in MHz or KHz depending on the configuration.

It is also possible to use the mouse wheel to enter the tuning frequency.

## Mode group

These buttons determine the operation mode of the receiver.

## AGC group

These buttons determine the AGC used by the receiver.

## Filter group

One of the 34 different filter choices available with the RX-320 can be selected in this group.

The filters are grouped into 4 groups each containing 9 different filter values. The group can be selected with the 4 radio buttons at the left side while the actual filter can be selected by pressing one of the 9 buttons at the right side. (If the filter group is changed, the filter values displayed on the 9 buttons will be exchanged).

## Passband shift group

Passband shift can be applied in the modes USB, LSB and CW; it will not be operational in AM mode.

The slider allows to choose the passband shift. Passband shift will only be active if the check box at the right side is checked; in this case the value of the passband shift will be displayed (in Hz).

## Signal strength group

The signal strength will be displayed if the check box at the right side is checked. The top bar in the control will display the actual signal strength while the lower bar will show an average over the last probes.

The slider control below the display of the signal strength regulates the squelch level. Squelch suppression will only be active if i) display of the signal strength is chosen (checkbox checked) and ii) the squelch suppression is chosen (squelch checkbox checked).

The state of the squelch suppression will be shown in the small display in the lower left corner of this group.

## Station group

The list control in this group displays all the stations which are available in the station file currently selected. The name of this station file is displayed in a display in the lower right corner of the station group.

The listbox shows the frequency and the name (description) of the stations. Clicking the header ”Frequency” will sort all the entries by ascending frequency; clicking the header ”Station” will sort the entries by station name.

A single entry in the listbox can be selected by clicking it. Pressing the ”Tune” button will tune the receiver to the selected station. Tuning the receiver to a selected station can also be achieved by doubleclicking the entry.

Pressing the button ”Add” will create a new station entry in the list control: a dialog box will be displayed where the current settings of the receiver are shown. A station name can now be entered and optionally the settings can be altered. The new entry will be created if the button ”Ok” is chosen to leave the dialog.

The ”Update” button will update the stored settings of the selected station with the current radio settings, i.e. everything besides the station name is replaced with the actual radio settings. The values are entered into an edit dialog so that they may be altered by the user.

With the ”Edit” button an existing entry in the station list can be modified. The dialog box used for that purpose is the same as the dialog encountered when adding or updating an entry.

Pressing the ”Delete” button will delete the selected entry in the station list. In order to prevent the user from accidentally deleting entries, this operation must be confirmed.

## Volume group

The slider available in this group regulates the audio level available at the audio output of the RX-320. Unchecking the check box at the right side of this group will turn the output completely off (”Mute”).

## Line out group

The function of the controls in this group is precisely the same as in the volume group with the difference that now the audio level at the line output is regulated.

## Display of time and date

The actual date and UTC time are displayed in the following format:

dd.mm.yyyy   hh:mm:ss  UTC

## Station file button

This button belongs logically into the station group. When the button is pressed, a sub-dialog is displayed from which the actual operation ”New”,  ”Open”, ”Save” or ”Save as” can be chosen. Accordingly a new, empty station file will be generated, an existing station file can be opened and displayed, the actual station file can be saved or can be saved with another name.

## PDB button

The PDB (program database) button is only relevant in conjunction with the ILG broadcast database program GNPDB.

If the PDB button is clicked, the actual receiver frequency is used to look up the ILG database. The cursor in the database program is positioned to the first station entry with that frequency.

## Configure button

Clicking the configure button will display a mask where several system settings can be configured:

- COM port :			the number of the COM port used for the connection to

the hardware.

- frequency display :		the tuning frequencies may be input and displayed

in MHz or KHz.

- frequency correction :	correction frequencies can be entered for two frequencies; the actual frequency correction for a certain tuning frequency is then calculated via linear interpolation.

- CW frequency :		determines the CW tone; may be positive or negative.

- tuning time slow and fast :	the time the program uses between two consecutive

changes of the tuning frequency in continuous tuning

mode.

## Exit button

The program will be stopped when this button is clicked. The user can choose whether he wants to mute the radio when exiting.

# Controling the program via keyboard

In addition to the normal Windows-like operation via mouse almost all of the functions of the program GNRX320 can be invoked via keyboard input.

Keyboard control is invoked by entering one of the characters shown in the table below. In addition the shift-key or the ctrl-key may be pressed simultaneously.

If a digit (”0” – ”9”) or a period (”.”) is entered, this is interpreted as a frequency input.

| Function | Char | Normal (lowercase) | Shift (uppercase) | Ctrl |
| --- | --- | --- | --- | --- |
| Up / Down | T | Frequency down | Frequency up | - |
| ContUp / ContDown | C | Cont Down | Cont Up | Stop cont. tuning |
| Fast | Y | - | - | Toggle fast / slow |
| Frequency input | 0 – 9 and . | Frequency input | - | - |
| Enter | Z | Choose | - | - |
| Step size | S | Down | Up | - |
| Mode | M | Down | Up | - |
| AGC | A | Down | Up | - |
| Filter Group | G | Down | Up | - |
| Filter | F | Down | Up | - |
| Passband shift | P | Down | Up | Toggle on/off |
| Signal strength | B | - | - | Toggle on/off |
| Squelch | Q | Down | Up | Toggle on/off |
| Station list | O | Set focus | - | - |
| Tune | X | Choose | - | - |
| Add | N | Choose | - | - |
| Update | U | Choose | - | - |
| Edit | E | Choose | - | - |
| Delete | D | Choose | - | - |
| Line out | L | Down | Up | Toggle on/off |
| Volume | V | Down | Up | Toggle on/off |
| PDB | K | Choose | - | - |
| Configure | R | Choose | - | - |
| Station file | I | Choose | - | - |

# Appendix

## Format of the station file

The GNRX320 station file uses  ASCII format to store the entries; it has the file extension ”tts”. The file has the following format:

- Line:

240951

This is a magic number which is used to distinguish a GNRX320 station file from other ASCII files which have the same extension ”tts” .

- Line:

1

This is the version number of the station file format (currently 1).

- Line and following lines:

<station name> | <frequency> | <mode> | <AGC> | <filter index> | <passband shift>

One line is used per station; the different values in a line are separated by  a ”| ” sign.

The station name can be of arbitrary length. If no station name is given, at least one blank must be supplied.

The frequency is given in Hz.

Mode may be:   0 = AM, 1 = USB, 2 = LSB, 3 = CW.

AGC may be:    1 = Slow, 2 = Medium, 3 = Fast.

The filter index is any value between 0 and 34 with the following meaning:

Filter index = 0 is the smallest possible value (300 Hz), filter index = 34 the largest possible value (8000 Hz).

The passband shift is given in Hz; it may vary between –1000 Hz and +1000 Hz.

## Configuration of the lowest tunable frequency

This paragraph is irrelevant if a standard unmodified RX-320 receiver is used. It is only relevant if a modifed RX-320 is used which can receive frequencies below 100 kHz.

In order to allow the control program GNRX320 to tune to frequencies below the standard

100 kHz, the lowest tunable frequency can be set in the file gnrx320.ini.

In the ”configuration” section of the file gnrx320.ini an entry ”MinimumFrequency” must be added with an editor (e.g. notepad):

[Configuration]

.

.

MinimumFrequency=xxxxx

.

.

where xxxx is the lowest tunable frequency in Hz (must be >= 10000 and <= 100000).